Temperature Sensing

Product Catalogue

Professional · Quality · Innovation

Thermocouple Fundamentals

A thermocouple uses two different conductors to measure temperature through the thermoelectric effect. Thermocouple types have

different EMF outputs and operating conditions; reference-junction temperature compensation is required.

Common Thermocouple Types

K NiCr / NiSi

E NiCr / constantan

J Iron / copper-nickel

T Copper / constantan

N NiCrSi / NiSiMg

Wide range and good oxidation resistance; widely used in furnaces, heat treatment and

process measurement. Protect in reducing atmospheres.

High EMF output for sensitive measurement. Suitable for oxidizing or inert atmospheres

and low temperatures.

General industrial use. The iron leg can rust in damp conditions; protect it in

high-temperature oxidizing atmospheres.

Good low-temperature stability for low and cryogenic temperatures. Use suitable

protection for prolonged service.

Good high-temperature oxidation resistance and long-term stability. Select for the

atmosphere and protective construction.

Thermocouple EMF and Temperature

EMF / mV

Temperature / °C

Reference junction: 0°C. Curves use NIST ITS-90 reference functions and show non-negative temperatures only. Curve endpoints are not product ratings; service limits depend

on wire diameter, sheath material, atmosphere and installation.

Changzhou OSK Special Cable Co., Ltd.

Contents

We develop and manufacture precision temperature

sensing and heating products. Advanced technology,

strict quality control and comprehensive service

support reliable, stable solutions for metallurgy,

chemicals, energy, aerospace, laboratories and

automation, helping customers operate efficiently,

safely and sustainably.

About Us

Quality Assurance

Applications and Delivery

Principles and Products

Temperature Data and Fine MI

Special Thermocouples

Miniature Heaters

Metals and Special Materials

Thermocouple Wire

Platinum RTDs

Instruments and Accessories

About Us

Changzhou OSK Special Cable Co., Ltd.

Established in the 1980s as the former Xinhe Special Wire Factory in

Lucheng, Changzhou, we have over 40 years of experience in special

mineral insulation and precision thermocouples.

Decades of development have built an integrated factory covering MI stock

drawing, high-density insulation filling, custom structural design and

finished thermocouple production.

We serve demanding supply chains in semiconductor process control,

medical devices, aerospace, defense research and advanced chemicals,

where temperature, insulation and pressure performance are critical.

Factory aerial view

Factory building

Why OSK

Independent R&D, standardized management, supplier

qualifications and technical patents support our role as a

qualified supplier to defense organizations, universities and

multinational manufacturers.

Independent R&D and patents

Qualified defense supplier

University and multinational supplier

Certified quality management

Milestones

Company founded

Focus on temperature sensing and specialized

measurement products.

Production line built

Our first independently developed MI line enabled

in-house, scaled thermocouple production.

Sales channels expanded

Online sales launched, with improved company

structure, sales channels and customer service.

Supplier partnerships expanded

Supplied PLA Factory 6456, Xi’an Jiaotong University,

AVIC and AECC for research, aerospace, defense and

advanced equipment.

Capacity expanded

Factory expansion improved production scale,

delivery efficiency and custom manufacturing.

Stable supply across industries

Supplied the Beijing Institute of Satellite Environment

Engineering, NUDT and other customers in defense,

aerospace, semiconductors, medical devices and new

energy.

Temperature sensing for advanced manufacturing

Company Overview

Core capabilities

Products and focus

Technology and processes

Quality and service

• In-house design and manufacture of MI wire drawing and precision

annealing lines.

• High-density MgO filling for uniform, stable insulation.

• An in-house laboratory with high-voltage insulation tests, X-ray inspection

and precision electrical test systems.

• Fine-diameter MI materials with heat resistance, fast response and strong

interference resistance.

• Multiple drawing specifications, sheath materials and precision

fine-diameter MI thermocouples.

• Semiconductor, precision sensing and new-energy equipment

applications.

• Fine-wire drawing with mature processes and tight dimensional accuracy.

• Fine thermocouples for fast, accurate measurement.

• Custom thermocouples and parts for special conditions.

• Nickel-clad copper balances conductivity, heat and oxidation resistance for

cost-sensitive electrical uses.

• Industrial sensing wire for complex environments.

• Standardized production throughout, with products meeting international

standards.

• Core products used in aerospace, semiconductors, medical devices and

precision manufacturing.

• Technical support and custom solutions.

• Continuous improvements for demanding global markets.

Quality Assurance

Quality as our foundation Innovation as our drive Customer focus

Awards and Qualifications

High-tech enterprise certificate

Specialized and innovative SME

Management System Certification

Quality management

Occupational health and safety

High-tech product certificate

Environmental management

Quality Assurance

Quality as our foundation Innovation as our drive Customer focus

Test Reports

Calibration certificate

Test report

Verification certificate

Selected Customers

Beijing Institute of Satellite

Environment Engineering

National University of

Defense Technology

PLA Factory 6456

Forcecon Technology Co., Ltd.

Shenzhen S.C New Energy

Technology Corporation

Shanghai Meili Weiye

Medical Technology Co., Ltd.

Product Applications

Precision temperature measurement and signal transmission for demanding industrial applications requiring heat resistance,

accuracy, fast response and interference resistance, supported by an agile supply chain.

Aerospace and Power

For extreme temperature differences and vibration.

New Energy and Photovoltaics

Precision sensing throughout battery and PV production.

Semiconductors and Precision Manufacturing

Temperature control for chips and precision instruments.

Automotive and Transport

Thermal management for engines and new-energy powertrains.

Medical and Life Sciences

Sensitive sensing for minimally invasive procedures and devices.

Industry and Food Processing

Food-grade probes and industrial temperature sensing.

Inventory and Delivery

Stock: dispatch in 24 h

Assessment: reply in 24 h

100% final inspection

Fast Stock Dispatch

Standard products held in stock with systematic

inventory control. Dispatch within 24 hours after

order confirmation helps keep production

running.

Efficient Custom Orders

Our technical team assesses requirements

promptly. Flexible scheduling and supply-chain

coordination support on-time custom delivery.

Priority for Urgent Orders

Priority handling and dedicated follow-up

shorten lead times and respond quickly to

urgent requirements.

Quality at Every Stage

Strict inspection from raw materials to finished

products, including 100% final inspection,

supports reliable product quality.

Managed Warehouse

Zoned storage · ample stock · first in, first out

Accurate inventory and efficient turnover

How a Thermocouple Works

Temperature sensing solutions

A thermocouple uses two dissimilar metal conductors. A temperature difference between the measuring and reference junctions

generates an EMF. The instrument calculates temperature from this voltage and the reference-junction temperature using cold-junction

compensation.

Operating principle

Measuring (hot)

Two different metal wires

join at the measuring end.

Temperature and Voltage

Voltage (mV)

Reference (cold)

Instrument-terminal temperature

must be known or measured.

EMF output

Small voltage generated by

the

temperature difference.

Measured temperature

Hot junction

100°C

Reference temperature (cold junction)

0°C in this example; actual instruments

provide cold-junction compensation.

Temperature (°C)

Type K; reference 0°C; approximately 4.10 mV at 100°C

Test Equipment

High-temperature calibration furnace

Constant-temperature oil bath

Dry-well calibrator

Microfocus X-ray inspection system

Insulation resistance tester

Precision temperature logger

100°C ≈ 4.096 mV

Mineral Insulated

Thermocouples

About MI Thermocouples

An MI thermocouple combines thermocouple conductors, insulation and a

metal sheath into a robust, drawn assembly.

Compact and durable, MI thermocouples work with transmitters,

controllers and displays for process measurement and control.

Service temperature depends on type, sheath, probe and

conditions. A range of 0-1800°C does not apply universally to all MI

thermocouples.

Conductors and Materials

• Thermocouple wire: K, T, J, E, N, R, S, B

• Insulation: alumina or magnesium oxide; MgO is most widely used.

• Sheath: 304, 316L, 310S, Inconel 600,

GH3039, GH3030, Hastelloy and others.

Enlarged MI cross-section

Three Common Junction Types

Exposed junction

Fast response for applications such as engine exhaust;

lower mechanical strength.

Grounded junction

Junction welded to the sheath for fast response.

Assess grounding and noise. Nominal pressure

reference: 34 MPa; confirm by model and conditions.

Ungrounded junction

Electrically isolated from the sheath to reduce

ground-loop and noise effects. Usually slower than an

equivalent grounded junction.

Exposed

Grounded

Ungrounded

TC wire

MgO powder

Metal sheath

MI Probes and Mounting Options

Conductors, insulation and sheath form a drawn assembly. Bending must meet the requirements of the specific probe construction.

MI Probe Construction

Thermocouple conductors, insulation-grade MgO and a metal

sheath form one assembly; shape to the specified minimum

bend radius.

Metal sheath

Insulation-grade MgO

Mounting Options

No fixed fitting, fixed thread, adjustable compression fitting or flange.

Select mounting position and sealing for the operating conditions.

Conductor

Probe may be formed as specified

Reference bend radius ≥3D

No fixed fitting

Convenient for spot

measurement.

Fixed thread

Custom thread; fixed position.

Compression fitting

Adjustable insertion depth;

custom thread and ferrule.

Flange

Fixed, sealed mounting; custom

flange size.

Default probe: 6 × 40 mm;

custom sizes available.

Flexible strain relief protects the lead.

Electrical Terminations

Available MI thermocouple types can use plugs, terminals, pins or bare leads with compatible measuring instruments.

Plug

Portable instruments

Terminals

Transmitters and controllers

Pins

Recorders and PLCs

Bare leads

Transmitters and controllers

MI Thermocouple Thermal Response Reference

Thermal Response Time

Notes

1. Boiling-water tests; values are response-time references.

2. Confirm t50 / t63.2 / t90, immersion and flow conditions in the test report.

3. For equivalent structures, smaller diameters and grounded junctions usually

respond faster.

MI Probe Diameter and Test Parameters

Confirm AC/DC test format, duration and acceptance criteria against the product test specification.

| MI outside diameter mm | Grounded response s | Ungrounded response s | Temperature step °C | Test conditions |
| --- | --- | --- | --- | --- |
| 0.25 | 0.007 | 0.012 | Ambient to 100 | Boiling water |
| 0.5 | 0.027 | 0.031 | Ambient to 100 | Boiling water |
| 0.5 | 0.030 | 0.050 | Ambient to 100 | Boiling water |
| 1.0 | 0.077 | 0.117 | Ambient to 100 | Boiling water |
| 1.0 | 0.070 | 0.120 | 0 to 100 | Boiling water |
| 1.6 | 0.150 | 0.200 | 0 to 100 | Boiling water |
| 1.6 | 0.180 | 0.260 | Ambient to 100 | Boiling water |
| 2.3 | 0.260 | 0.410 | Ambient to 100 | Boiling water |
| 3.2 | 0.400 | 0.500 | 0 to 100 | Boiling water |
| 3.2 | 0.460 | 0.900 | Ambient to 100 | Boiling water |
| 4.8 | 0.730 | 1.200 | 0 to 100 | Boiling water |
| 4.8 | 1.600 | 2.400 | Ambient to 100 | Boiling water |
| 6.4 | 1.200 | 2.400 | 0 to 100 | Boiling water |
| 6.4 | 2.200 | 3.700 | Ambient to 100 | Boiling water |
| 8.0 | 2.100 | 3.900 | 0 to 100 | Boiling water |
| 8.0 | 4.000 | 5.800 | Ambient to 100 | Boiling water |

| Outside diameter D mm | Diameter tolerance mm | Withstand test V | Reference bend radius |
| --- | --- | --- | --- |
| 0.5 | ±0.025 | 100 | ≥3D |
| 1.0 | ±0.025 | 100 | ≥3D |
| 1.5 | ±0.025 | 100 | ≥3D |
| 2.0 | ±0.025 | 500 | ≥3D |
| 3.0 | ±0.030 | 500 | ≥3D |
| 3.2 | ±0.032 | 500 | ≥3D |
| 4.0 | ±0.040 | 500 | ≥3D |
| 4.5 | ±0.045 | 500 | ≥3D |
| 4.8 | ±0.048 | 500 | ≥3D |
| 6.0 | ±0.060 | 500 | ≥3D |
| 6.4 | ±0.064 | 500 | ≥3D |
| 8.0 | ±0.080 | 500 | ≥3D |
| 9.5 | ±0.095 | 500 | ≥3D |
| 10.8 | ±0.108 | 500 | ≥3D |

Recommended MI Thermocouple Temperatures

Unit: °C

Notes

1. Recommended values for conventional conditions, for selection reference only.

2. Actual limits depend on sheath, atmosphere, corrosiveness, installation and required life.

3. High-temperature Types S, R and B generally use alumina or ceramic tubes; a direct metal-sheathed MI

construction is generally not recommended.

Sheath Material Temperature Reference

Unit: °C

Notes

1. Conventional material reference values, rather than complete-probe ratings.

2. Confirm continuous or short-term temperature and grade; consider atmosphere, corrosion and life.

3. Verify the specific product limits for high temperature, severe corrosion, vibration or special atmospheres.

| Type | Ø0.25-0.5 mm | Ø1.0 mm | Ø1.5-3.0 mm | Ø4.5-6.0 mm |
| --- | --- | --- | --- | --- |
| T | 350 | 350 | 350 | 350 |
| J | 500 | 650 | 750 | 750 |
| E | 500 | 700 | 850 | 900 |
| K | 500 | 700 | 900 | 1,100 |
| N | 500 | 700 | 1,000 | 1,200 |

| Sheath | Ø0.25-0.5 mm | Ø1.0 mm | Ø1.5-3.0 mm | Ø4.5-6.0 mm |
| --- | --- | --- | --- | --- |
| 304 | 500 | 700 | 800 | 900 |
| 316L | 500 | 700 | 800 | 900 |
| 310S | 500 | 800 | 950 | 1,100 |
| Inconel 600 | 500 | 700 | 1,050 | 1,150 |
| GH3039 | 700 | 900 | 1,100 | 1,200 |
| Hastelloy | 500 | 700 | 900 | 1,000 |

Fine-Diameter

MI Thermocouples

Fine diameters · Custom designs · Dedicated manufacture

We develop and manufacture fine-diameter MI thermocouples for

confined spaces, complex structures and special conditions. Our

product-development and production experience supports custom

dimensions, construction and connections for your application and

installation.

Ø0.25 mm

Outside diameter down to

Key Advantages

Fine diameters

For confined spaces, precision equipment

and complex installations.

Custom manufacture

Custom diameter, length, type, sheath,

junction, shape, mounting and termination.

Manufacturing experience

Long-term expertise in product

development, processing and special

applications.

Fast response

Small dimensions and low thermal mass for

rapid temperature measurement.

Custom Options

Confirm available combinations of diameter, type and sheath for the specific model.

From a Thermocouple to a Complete Custom Solution

Requirements

Application analysis

Design

Sample approval

Production

Final inspection

Diameter

Ø0.25 / 0.5 / 0.8 / 1.0 / 1.5 / 2.0 mm ...

Type

K / J / E / T / N / S / R / B

Sheath

SUS304 / SUS316 / 310S / Inconel ...

Junction

Grounded / ungrounded / exposed / special

Construction

Straight / bent / preformed / threaded / compression /

special

Termination

High-temperature leads / compensating cable /

standard or miniature connectors / custom

Multipoint Thermocouples

Multipoint thermocouples measure several positions within one area.

Multiple sensing points are integrated into a protection tube or assembly

with different lengths and positions, for monitoring temperature distribution,

gradients and critical process points.

They collect more temperature data in limited space, reduce vessel

penetrations and simplify installation for more complete monitoring.

Sensing-point count and spacing, tube material, wiring and mounting can

be customized for high temperature, high pressure, corrosion and wear.

Used in reactors, tanks, towers, furnaces, piping, chemical plants and

power systems to support operating monitoring, process optimization,

safety interlocks and fault diagnosis.

Features

• Multiple points in one sensor to monitor temperature

distribution.

• Fewer equipment penetrations and simpler installation

and maintenance.

• Custom point count, position and length.

• Types K, J, E, N and T available.

• Tube materials for heat, corrosion and wear.

• Stable output for continuous process measurement.

Applications

• Temperature profiles in chemical reactors.

• Different heights in tanks and silos.

• Furnace and heat-treatment zones.

• Critical points in piping, towers and pressure

vessels.

• Power, metallurgy and petrochemical processes.

Measuring points

Point 1

Point 2

Point 3

Point 5

Single-point arrangement

Tube outside diameter

Protection tube length L1

L1: protection tube length excluding leads.

L2: lead length from the connection to the terminal end.

Type K MI

thermocouple

Protection tube

Multipoint arrangement

Lead length L2

Point 4

Ultrafine Thermocouple Wire

Hair-fine conductors

Surface sensing in small spaces

Minimally invasive procedures, electronic chips and lithium batteries

Thermocouple wire

Sensing wire

Connectors

Hair

Probe Options

Welded junction, foil, adhesive foil and small-tube probes for different temperature measurements.

Welded junction

Very small spaces

Foil

Surfaces or narrow gaps

Adhesive foil

Surface temperature

Small metal tube

Liquids or internal temperatures

Specifications

Sensing wire

Connectors

* Confirm the insulation grade and corresponding temperature range for the operating conditions. Under IEC 60584-1, Type T Class 1 tolerance applies from -40 to 260°C;

lower-temperature tolerances are specified separately.

| Parameter | Type K ultrafine wire | Type T ultrafine wire |
| --- | --- | --- |
| Type | K | T |
| Insulation | Polyesterimide / polyimide / polyester | Polyesterimide / polyimide / polyester |
| Temperature* | -20 to 200°C / -20 to 260°C | -200 to 200°C / -200 to 260°C |
| Tolerance | Class 1 | Class 1 |
| Conductors | NiCr (+) / NiSi (-) | Copper (+) / constantan (-) |
| Conductor diameter | 0.05-0.2 mm | 0.05-0.2 mm |
| Overall diameter | Custom | Custom |
| Enamel thickness | Custom | Custom |
| Enamel color | Custom | Custom |
| Termination | Custom | Custom |

Reduced-Tip Thermocouples

Applications

Temperature measurement in limited space while maintaining mechanical strength.

Manufacturing Principle

Local reduction of the sensing end uses plastic deformation to compress the sheath, MgO

insulation and thermocouple conductors together. It forms a continuous, uniform reduced

section while keeping the internal structure intact and preventing conductor breakage or

insulation failure.

Typical reductions

• Ø3.2 to Ø2.3 mm

• Ø6 to Ø3.2 mm

• Ø4.8 to Ø1.6 mm

• Ø3.0 to Ø1.6 mm

Custom sizes available

Design Considerations

Reduced-tip dimensions

Select body diameter, reduced diameter and reduced length for response time, installation space and

mechanical strength. Balance thermal response with structural strength.

Transition design

Use a smooth radiused or tapered transition to avoid abrupt section changes, reduce stress concentration and

improve bending and fatigue performance.

Manufacturability

Keep reduction within process limits to avoid sheath cracking, conductor damage or reduced insulation

performance.

Hot Runner

Thermocouples

1 Application

Temperature measurement in injection-mold hot

runners: nozzles, manifolds, hot tips and

confined spaces inside molds.

2 Features

Fast response

Accurate sensing

Flexible form

Heat stability

Specifications

• Ø0.25 / 0.5 / 0.8 mm

• Ø1.0 / 1.5 / 2.0 mm

• Types K / J / E / T / N

• Straight / bent / preformed

Custom

3 Selection

1 Probe size

Select diameter and

length for the hole

size and depth.

2 Tip structure

Select for response

speed and

installation.

3 Mounting

Select for available

space and lead

routing.

4 Connection

Select for cable

distance and

environment.

5 Materials

Select for

temperature and

service conditions.

Product Highlights

Fine probes for small spaces

Precision sensing for stable control

Flexible installation

Heat stability and

durability

For nozzles, manifolds, hot tips and mold

interiors

Miniature MI Heaters

Miniature mineral insulated heaters combine a heating element, insulation and

metal sheath through precision processing. Their compact, reliable structure

offers efficient heat transfer and fast response for uniform heating in confined

installation spaces.

Mechanical strength, high-temperature resistance and long-term stability suit

precision heating with demanding size, response and control requirements. Various

structures and custom designs support different equipment and operating conditions.

Features

Compact design

Flexible installation in space-limited equipment.

Robust construction

Reliable MI construction resists vibration and shock.

Fast response

Close element-to-sheath contact improves heat

transfer for rapid heat-up.

Stable control

For precision temperature control and continuous

heating.

Applications

Semiconductor equipment, analytical and test instruments, medical devices, precision molds, hot runners, automation, laboratories and

other processes requiring rapid heating and stable control.

| Parameter | Options |
| --- | --- |
| Sheath outside diameter | Ø0.5 / 1.0 / 1.5 / 2.0 / 3.0 / 4.0 / 5.0 / 6.0 mm; custom |
| Sheath material | SUS304 / SUS316L / SUS321 / SUS310S / Inconel 600, etc. |
| Heating wire | Cr20Ni80 (NiCr80/20) / iron-chromium-aluminum (FeCrAl) |
| Insulation | High-purity magnesium oxide (MgO) |
| Lead arrangement | Single-ended or double-ended; custom |
| Rated voltage | 12 / 24 / 36 / 48 / 110 / 220 / 380 V or custom |
| Power | Custom |
| Heated length | Custom |
| Overall length | Custom |
| Cold section length | Custom |
| Lead length | Custom |
| Lead types | Nickel-plated copper / nickel / fiberglass / silicone / PTFE |
| Lead exit | Axial / radial / 90° bend / custom |
| Mounting | Direct insertion / thread / flange / compression fitting |
| Seal | Epoxy / silicone / ceramic |

Nickel-Clad

Copper Wire

High conductivity High temperature resistance High strength

Composite construction

Product appearance

Nickel layer

Copper core

Specifications

Material: nickel-clad copper composite wire

Nickel: (30 ± 3)% (confirm the measurement basis in the specification)

Copper core: balance, approximately 67%-73%

Diameter: 0.15-8.0 mm

Features

Low resistivity

High conductivity

Corrosion resistance

High-temperature oxidation resistance

High tensile strength

Strong nickel-to-copper bond

Material advantages

Copper core

High conductivity

Low resistivity

Good ductility

Nickel layer

High-temperature resistance

Oxidation resistance

Corrosion resistance

Applications

Thermocouples and sensing

Automotive systems

High-temperature cables

Industrial equipment

Heating elements

Special electrical connections

Pure Silver Wire

High purity · High conductivity · Reliable · Custom

1 Application

Electronics, sensors, precision instruments, aerospace,

advanced connections and industrial measurement.

2 Features

Conductivity

High purity

Ductility

Reliability

3 Specs

• Diameter: custom.

• Purity: various silver grades to suit

requirements.

• Conductor: solid, stranded or special.

• Length and packing: cut lengths, spools or

coils.

• Custom designs for temperature, conductivity

and other requirements.

4 Applications

Electronics

Sensors

Precision

Aerospace

Connections

Industry

5 Custom Process

Requirements

Material selection

Specifications

Precision processing

Testing

Delivery

Product Highlights

High-purity silver for excellent conductivity

Good ductility for complex processing

Stable and reliable in diverse applications

Custom manufacture and prompt response

Thermocouple Extension

and Compensating Cable

Types K / T / J / N / R / S and others

Extension vs. Compensating Cable

Extension cable (e.g. KX) uses the same nominal conductor alloys

as its thermocouple. Compensating cable (e.g. KCA/KCB) uses

different alloys with matching thermoelectric characteristics within a

specified temperature range.

Both carry the thermocouple signal to the instrument. Match type,

polarity and temperature range correctly. Cold-junction

compensation is performed by the instrument, rather than

automatically by the cable.

Construction: conductors, insulation, jacket and optional shield.

Metal A

Measuring junction

Instrument

Metal B

Connections

Instrument cold-junction

compensation

Cable Selection and Connection

Match the cable to the thermocouple type for matching

EMF characteristics within the specified range.

Connect the correct polarity. Keep both connection pairs at

the same temperature.

Keep the cable and thermocouple connection within the

specified cable temperature range.

Select conductor diameter for the instrument requirements.

Features

Quality alloys

Stable electrical properties and precise

EMF matching.

Reliable insulation

Heat and abrasion resistance with good

electrical insulation.

Optional shielding

Select for the electromagnetic environment

to reduce noise.

Examples

Thermocouple, Cable and Connector Color Codes

KPS · ANSI · IEC

Connectors

Connectors

ANSI

Code

ANSI / ASTM E-230

Color code · TC grade

Ext. grade

IEC

Code

IEC 584-3

Color code · TC

grade

Ext. grade

Undefined

Undefined

Undefined

Undefined

Undefined

Undefined

Undefined

No IEC standard;

use ANSI colors

Alloy pair

Notes

Bare-wire atmosphere

Maximum

TC-grade range

NiCr

NiAl

(magnetic)

Cu

CuNi

constantan

Fe

(magnetic)

CuNi

constantan

NiCr

CuNi

constantan

NiCrSi

NiSiMg

Pt-13Rh

Pt

Pt-10Rh

Pt

Pt-30Rh

Pt-6Rh

W-5Re

W-26Re

Fe

(magnetic)

Clean oxidizing or inert atmospheres; limited vacuum

or reducing use. Wide range; most common type.

Mildly oxidizing, reducing, vacuum or inert. Good in

damp conditions and for low-temperature or

refrigeration use.

Reducing, vacuum or inert atmospheres; limited

high-temperature reducing use. Not recommended

for low temperatures.

Oxidizing or inert; limited vacuum or reducing use.

Highest EMF change per degree.

Alternative to Type K; suited to higher temperatures.

Oxidizing or inert. Avoid metal tubes and

contamination. High-temperature use.

Oxidizing or inert. Avoid metal tubes and

contamination. High-temperature use.

Oxidizing or inert. Avoid metal tubes and

contamination. High-temperature and glass

applications.

Vacuum, inert or hydrogen. Protect from oxidation.

Impractical below 399°C (750°F); unsuitable for

oxidizing atmospheres.

-200 to 1372°C

-270 to 400°C

-210 to 1200°C

-270 to 1000°C

-270 to 1300°C

0 to 1600°C

0 to 1600°C

0 to 1800°C

0 to 2320°C

Insulation and Jacket Temperature References

Customization Options

Thermocouple Type and Application Reference

+ leg

- leg

| Material | Range °C | Features | Applications |
| --- | --- | --- | --- |
| PVC | 70-105 | Low cost; flexible | Ambient instruments; cabinet wiring |
| Silicone | 180-200 | Flexible; bend and thermal-shock resistance | Heaters; molds; moving connections |
| PTFE | 200-260 | Heat, chemical and insulation performance | Chemical; semiconductor; precision equipment |
| PFA | 200-260 | Chemical resistance; smooth and clean | Semiconductor; medical; clean equipment |
| Fiberglass | 350-500 | Heat and aging resistance | Ovens; furnaces; heat treatment |
| Fiberglass + shield | 350-400 | EMI, abrasion and heat resistance | Industrial furnaces; strong EMI |
| Ceramic fiber | 600-1000 | High heat resistance; lower flexibility | Furnaces; kilns; hot zones |
| Quartz fiber | 500-800 | Heat and thermal-shock resistance | High-temperature and heat-treatment furnaces |

| Parameter | Available options |
| --- | --- |
| Conductor | Solid / stranded |
| Core count | 2 / 4 / 6 / 8 / custom multicore |
| Diameter | Ø0.2 / 0.3 / 0.5 / 0.8 / 1.0 / 1.2 mm, etc. |
| Cross-section | 0.2 / 0.3 / 0.5 / 0.75 / 1.0 / 1.5 mm², etc. |
| Insulation | Single / double layer / insulation plus jacket |
| Shield | None / tinned copper / stainless braid / foil / overall / individual |
| Jacket | PVC / silicone / PTFE / PFA / fiberglass / metal braid / ceramic fiber |
| Color standard | IEC / ANSI / GB / customer specified |
| Tolerance | Class 1 / Class 2 / special |
| Packing | Coils / spools / cut lengths / prepared ends |

| Type | Conductors | Typical cable | Sensor range °C | Main characteristics | Typical applications |
| --- | --- | --- | --- | --- | --- |
| K | NiCr / NiSi | K; KX extension; KCA/KCB compensating | -200 to 1260 | Versatile; widely used | Heaters; molds; furnaces; piping; machinery |
| J | Iron / CuNi | J wire; JX extension | -40 to 750 | Common at low/medium temperatures; low cost | Plastics; packaging; ovens; older controllers |
| T | Copper / CuNi | T wire; TX extension | -200 to 350 | Good low-temperature stability | Cold chain; laboratories; food; low temperatures |
| E | NiCr / CuNi | E wire; EX extension | -200 to 900 | High EMF and sensitivity | Laboratories; precision sensing; low-signal uses |
| N | NiCrSi / NiSiMg | N wire; NX/NC cable | -200 to 1300 | Heat stability; oxidation resistance | Furnaces; heat treatment; aerospace; advanced equipment |
| S | Pt-10Rh / Pt | S wire; SC compensating | 0 to 1600 | High accuracy and stability; higher cost | Lab and calibration furnaces; glass; ceramics; metallurgy |
| R | Pt-13Rh / Pt | R wire; RC compensating | 0 to 1600 | High-temperature accuracy and stability | Industrial furnaces; laboratories; noble-metal thermometry |
| B | Pt-30Rh / Pt-6Rh | B wire; BC compensating | 600 to 1700 | Very high temperatures; low sensitivity at low temperatures | Metallurgy; glass and ceramic kilns; furnaces |

Ceramic fiber

White / red dot: NiAl

White / yellow dot: NiCr

Ceramic-fiber jacket

Metal shield

Prevents fiber

spreading

NiSi (-)

NiCr (+)

NiSi (-)

NiCr (+)

Stainless-steel

shield

Stainless-steel shield

Solid

conductor

Stranded

conductor

Copper (+)

NiCr (+)

Constantan

(-)

Brown PTFE jacket

NiSi (-)

Polyimide jacket

NiAl (-)

NiCrSi (+)

Red PTFE insulation

PTFE film

NiCr (+)

Brown PTFE jacket

NiSiMg (-)

Gray PTFE

insulation

Metal shield

NiSi (-)

NiCr (+)

Black PTFE

insulation

PTFE film

Red PTFE insulation

Clear PTFE jacket

NiAl (-)

NiCr (+)

White PTFE insulation

Red PTFE insulation

Blue PTFE jacket

Red PTFE insulation

Blue PTFE jacket

Copper (+)

Constantan

(-)

Blue PTFE insulation

Blue PTFE jacket

Brown PTFE jacket

White PTFE insulation

Red PTFE insulation

Red PTFE insulation

Wire Ø0.3 mm

White PTFE insulation

Constantan

(-)

Wire Ø0.5 mm

Iron (+)

NiAl (-)

NiCr (+)

Green PTFE jacket

Green PTFE jacket

White PTFE insulation

NiAl (-)

NiCr (+)

Green PTFE

insulation

Green PTFE

insulation

NiCr (+)

NiAl (-)

White PTFE insulation

Solid

Stranded

Black fiberglass

insulation

Red fiberglass

insulation

Black

fiberglass

insulation

Red fiberglass

insulation

Blue

translucent

fluoropolymer

insulation

Brown polyimide

insulation

Red translucent

fluoropolymer

insulation

Black polyimide

insulation

Red fluoropolymer

insulation

Yellow translucent

fluoropolymer

insulation

Fiberglass jacket

White PTFE insulation

Red fiberglass insulation

Red PTFE insulation

Clear PTFE jacket

Brown PTFE jacket

Copper (+)

Constantan (-)

Copper (+)

Constantan (-)

Constantan (-)

Iron (+)

Constantan (-)

Iron (+)

4 cores

Clear PTFE jacket

Silicone jacket

Red PTFE insulation

White PTFE insulation

Red PTFE insulation

White PTFE insulation

Clear PTFE insulation

Red PTFE insulation

Metal shield

Wire: 0.12 mm × 7 × 3

PTFE film

PTFE insulation

Tinned copper

Clear PTFE jacket

Silver-plated shield

OD approx. 2.8 mm

PTFE film

0.15 mm × 7 × 4

6 cores

PTFE jacket

Clear PTFE insulation

Red PTFE insulation

0.15 mm × 7 × 3

PTFE jacket

OD 3 mm

White PTFE insulation

Metal shield

Red PTFE insulation

0.15 mm × 7 × 3

OD 3.3 mm

Dense stainless-steel braid

Fiberglass insulation

OD 3.5 mm

Dense stainless-steel braid

Fiberglass insulation

Unshielded

0.15 mm × 7 × 3

OD 3 mm

0.15 mm × 7 × 4

OD 4 mm

Clear PTFE insulation

0.15 mm × 7 × 3

Red PTFE insulation

0.15 mm × 7 × 6

3 cores

0.2 mm × 7 × 3

4 cores

0.2 mm × 7 × 4

Shielded

0.15 mm × 7 × 3

OD 4 mm

Platinum RTDs

What is an RTD?

A resistance temperature detector (RTD) measures temperature through changes in electrical

resistance. Platinum RTDs offer high accuracy and good long-term stability for industrial processes and

calibration. The actual temperature range and tolerance depend on the element and probe construction.

Three RTD Lead Configurations

2-wire: one lead connects to each end of the platinum element. The reading includes lead resistance,

with error affected by lead length, material and ambient temperature. Suitable for short leads or

measurements that can accept the resulting error.

3-wire: one end has one lead and the other has two. An appropriate measurement circuit and

matched lead resistances reduce the influence of lead resistance. Common in industrial process

measurement.

4-wire: two leads connect to each end. Two provide excitation current and two measure the voltage

across the element (Kelvin connection), substantially reducing lead-resistance error for high-accuracy

measurement.

Platinum RTD Elements

Fast response

Element tolerance options

A, B, 2 × B, 1/3B, 1/10B. Confirm fractional tolerances and

applicable temperature ranges for the specific element.

Pt100 versus Pt1000

1 Resistance sensitivity

For the same temperature coefficient, the change per °C of

Pt1000 is 10 times that of Pt100. For α = 0.00385/°C, averages

over 0-100°C are:

Pt1000: 3.85 Ω/°C

Pt100: 0.385 Ω/°C

Resistance sensitivity is not thermal response speed.

2 Operating temperature

Determined by element construction, probe design and service

conditions, rather than nominal resistance alone.

Pt100 Tolerance Reference

Permissible deviation (±°C)

1/3DIN and 1/10DIN are fractional tolerances. Values are references; confirm the applicable temperature range and test conditions for the specific element.

| Temperature °C | Class A | 1/3DIN | 1/10DIN |
| --- | --- | --- | --- |
| -100 | 0.35 |  |  |
| -50 | 0.25 | 0.18 |  |
| 0 | 0.15 | 0.10 | 0.03 |
| 100 | 0.35 | 0.27 | 0.08 |
| 200 | 0.55 | 0.43 |  |
| 250 | 0.65 | 0.52 |  |
| 300 | 0.75 |  |  |
| 350 | 0.85 |  |  |
| 400 | 0.95 |  |  |
| 450 | 1.05 |  |  |

Direct-Insertion MI RTDs

Range: -200 to 500°C*

Details

Hexagonal fixing

Secures the probe against slipping or

loosening

304 stainless-steel sheath

316L available

Imported

platinum

element

Three-pin RTD plug with U-type copper contacts

Standard round-pin plugs and custom probe diameters available

Platinum element

Custom probe length

* Confirm temperature range and tolerance for the actual element and probe configuration.

Formable MI Probes

Conductors, insulation and sheath form a drawn MI assembly. Follow the bend radius

for the specific construction and avoid repeated bending. No fixed fitting is standard;

fixed threads or adjustable compression fittings are optional.

Fixed threaded fitting

Adjustable compression

fitting

Cold and heat resistant

Corrosion and wear

resistant

Water and oil resistant

Fast response

| Product | Direct-insertion MI RTD | Brand | OSK |
| --- | --- | --- | --- |
| Element | Pt100 default / Pt1000 custom | Temperature | -200 to 500°C* |
| Sheath | 304 stainless steel; 316L optional | Dimensions | Custom diameter and length |
| Tolerance | A / B / 1/3B / 1/10B | Application | Furnace and oven sensing |

Thermocouple Plugs and Sockets

SMPW Miniature Connectors

Types: N / T / K / R / S / E / J / U / C; M = male, F = female

Temperature: -29 to 180°C (-20 to 356°F)

Features

• Rugged, heat-resistant construction for -29 to 180°C.

• Solid or stranded wire up to 20 AWG.

• Fast connection and reliable contact.

• Wire divider; Phillips/slotted screws.

Structure

Wiring example

N

C

T

K

R / S

E

J

K

J

E

U

U

OSTW Standard Connectors

Types: K / T / J / E / N / R / S / U / C; M = male, F = female

Temperature: -29 to 180°C (-20 to 356°F)

High-Temperature Ceramic Connectors

Miniature and standard ceramic connectors

Temperature: -29 to 650°C (-20 to 1200°F)

Silicone insulation washer

Serrated male

contact

Glass-filled

nylon housing

Terminals

Wire divider

Reversible write-on window

Tapered male contacts

Glass-filled nylon housing

Wire divider

Terminals

Silicone insulation washer

Tapered male contact

Terminals

Color dot

High-purity alumina housing

Features

• Versatile universal design.

• Compact, for wire up to 14 AWG.

• Fast connection and reliable contact.

• Wire divider; Phillips/slotted screws.

Structure

Features

• Ceramic construction: -29 to 650°C reference range.

• Strong, durable ceramic housing.

• Screw terminals for easy wiring.

• Solid or stranded wire up to 20 AWG.

• Phillips/slotted screws.

• Suitable for vacuum furnaces.

Images for reference. Confirm available types, ordering codes and 180°C / 650°C rating conditions for the specific product.

T

C

T

C

N

R / S

Stainless-Steel Compression Fittings

Threads: M / G / ZG / NPT and others

Adjustable TC Fittings

1 Nut

2 Copper ferrule

* Confirm pressure and temperature for the specification and medium; the upper limits do not necessarily apply

simultaneously.

Applications

For MI thermocouples, probe RTDs and platinum-rhodium thermocouples.

Installation Diagram

Thermocouple probe

Mold

| Parameter | Specification | Parameter | Specification |
| --- | --- | --- | --- |
| Product | Stainless-steel fitting | Brand | OSK |
| Pressure | 15 MPa* | Temperature | ≤450°C* |
| Material | 304 default; 316 optional |  |  |
| Media | Water / oil / gas. Check material for corrosive media. |  |  |
| Threads | M / ZG / G / NPT; custom |  |  |
| Probe OD | 0.5 / 1 / 1.5 / 1.6 / 2 / 2.5 / 3 / 3.2 / 4 / 5 / 6 mm ... |  |  |
| Parts | Body / ferrule / nut; custom weld-on base |  |  |

Precision Temperature Instruments

Thermocouple Thermometers

Platinum RTD Thermometers

Thermocouple Thermometers

Types K / T / J supported

Single / dual / four channels

610: 1 channel

610L: 1 channel with logging

620: 2 channels

620L: 2 channels with logging

640: 4 channels

640L: 4 channels with logging

Probe input

LCD

Multifunction key 1

Switch type / hold reading

Power / backlight

Multifunction key 2

Maximum / minimum / average

Switch °C / °F

Applications

Manufacturing

Research

HVAC

Maintenance

High precision

Multiple sensor types,

accurate measurement

and fast response.

Channel options

Single, dual or four

channels for different

applications.

USB port

Battery

Data logging

Record and review data

for traceability and

analysis.

Rugged design

A slip-resistant, durable

housing for industrial

environments.

Let’s Talk About Your

Temperature Measurement Needs

From operating conditions to product specifications,

we will work with you to confirm each requirement.

Send an application description, drawing or sample

to discuss product selection and customization.

Changzhou OSK Special Cable Co., Ltd.

Contact Us

Mr. Fang +86 13806110632

Mr. Du +86 13806123888

Address: Xingchen Village, Hengshanqiao Town, Changzhou, Jiangsu, China

Website: www.czosk.com

Email: lihua@czosk.com

jiacheng@czosk.com